Correcting laser scanning intensity to distinguish rock types in a cave environment: a case study of the Gouffre Georges Cave, France

Michaela Nováková1, Jozef Šupinský1, Michal Gallay1, Eric Ferré2, Riccardo Asti, Michel De Saint Blanquet, Flora Bajolet, Patrick Sorriaux4

Affiliations:

1Institute of Geography, Faculty of Science, Pavol Jozef Šafárik University in Košice, Jesenná 5, 04001 Košice, Slovakia

2School of Geosciences, University of Louisiana at Lafayette, Lafayette, LA 70504, USA

4Patrick Sorriaux Retired petroleum geoscientist TOTAL and Speleologist, 09400 Bédeilhac, France

    - Introduction

Laser scanning has demonstrated its extensive potential for landscape mapping over the last few decades (Eitel et al., 2017). Majority of applications focus on acquiring 3D coordinates to parametrise the three-dimensional (3D) structure of the landscape while smaller portion of published research makes use of the recorded intensity of the backscattered laser pulse. related to the surface parameters is recorded for each point position. As a result, a 3D point cloud is generated, capturing highly detailed morphology and specific spectral properties of the surface surrounding the laser scanner. The advantage of this active remote sensing technique has been employed in the cave mapping, where the complex morphology challenges studying the cave surface by direct contact or optical methods in a high spatial resolution (Gallay et al., 2015). Compared to the methods traditionally used in speleology and cave surveying, such as traversing with the compass or total station, laser scanning reduces the work time and is light-independent, unlike digital photogrammetry (Idrees and Pradhan, 2016a). The cave environment represents a well-preserved complex system, where the study of rock types and structural geologic features devoid of any vegetation is possible. 3D mapping and subsequent digital modelling of the surface of these hardly accessible outcrops enables the spatial observation, differentiation, and localization of the identified structures and lithological contacts (Triantafyllou et al., 2019).

In the last decade, terrestrial laser scanning (TLS) of caves has been deployed in a range of scientific applications, e.g. archaeology (Burens et al., 2014; Kruger et al., 2016; Landeschi et al., 2019), geology and geomorphology (Silvestre et al., 2015; Fabbri et al., 2017; De Waele et al., 2018), geomorphometry (Gallay et al., 2016), palaeontology (Santos Delgado et al., 2012; Citton et al., 2017), and biospeleology (Azmy et al., 2012; McFarlane et al., 2015). The acquired 3D geometry is also beneficial for mapping and evaluating the ice surface morphology change (Milius and Petters, 2012; Berenguer-Sempere et al., 2014; Šupinský et al., 2019), assessing the natural hazards and collapse susceptibility (Santo et al., 2017; Idrees and Pradhan, 2018), for general visualization and creation of a 3D cave model (Núñez et al., 2013, Cosso et al., 2014). Recent studies demonstrated the potential of using the TLS intensity for the cave environment research. Intensity-based 3D texture model allows the discernment of micro-morphologies, different speleogenetic phases, karstification processes, and identification of cave arts and engravings (Idrees and Pradhan, 2017). In biospeleology, the intensity data can be used for non-intrusive population surveys and species recognition for cave fauna, especially roosting bats (Azmy et al., 2012) or birds (Idrees and Pradhan, 2016b), with satisfactory results for the selected cave sections even without any radiometric processing or calibration.

Besides the surface properties, the intensity values are influenced by several factors, including sensor parameters, acquisition geometry, and atmospheric conditions during scanning (Kaasalainen et al., 2011; Tan and Cheng, 2016). The atmospheric effect on laser signal propagation is often neglected in TLS. The rationale is in the short scanning ranges within which the effect of the atmospheric attenuation is considered to be constant if the atmospheric conditions remain unchanged during scanning and the air transmittance is very high (Carrea et al., 2016). The content of the surface moisture also affects the recorded intensity values which decrease with increasing moisture for water absorbs the near-infrared wavelengths (Franceschi et al., 2009). Therefore, in an extensive cave environment with variable dimensions comprising narrow passages leading to grand domes and specific atmospheric conditions, the effect of the influencing factors is considerable. Moreover, the multi-scan TLS intensity data obtained by different acquisition campaigns are not directly comparable (Tan et al., 2016). For extraction intensity values proportional primarily to the scanned surface reflectance, other influencing factors should be eliminated. Kashani et al. (2015) provide a comprehensive overview of the processing methods of the intensity values mainly divided into theoretical correction models based on the radar equation (Höfle and Pfeifer, 2007; Wagner, 2010) and data-driven empirical models (Höfle and Pfeifer, 2007; Teo and Yu, 2015; Xu et al., 2017), both with specific limitations. Scanning geometry is defined by the scanning range and the incidence angle, i.e. the angle between the incident laser beam and the normal vector of the surface (Soudarissanane et al., 2011). Kaasalainen et al. (2011) showed that the effect of incidence angle is caused by the target surface properties, whereas the distance effect is mainly influenced by the instrumental factors and does not entirely follow the radar equation range function. Specifically, near-distance intensities up to several meters deviate from the radar range equation and vary across different laser scanning systems. Moreover, the correction of the incidence angle effect using Lambert´s cosine law is not sufficient, especially for the natural surfaces, which do not behave as Lambertian reflectors (Carrea et al., 2016; Tan et al., 2016).

Current remote sensing techniques offer high-resolution observations for mapping even hardly accessible geological outcrops. Recent studies have demonstrated the potential of the corrected TLS intensity data for geological mapping and distinguishing rock types within the aboveground rock outcrops (Penasa et al., 2014; Matasci et al., 2015; Carrea et al., 2016; Živec et al., 2019), without the need of eliminating the atmospheric attenuation effect. The presented paper aims to define the method of correcting the laser scanning intensity acquired in the cave environment, where the variable dimensions and limited light conditions restrict the use of optical sensing for material differentiation. The proposed intensity correction approach combines the data-driven methods for the distance effect correction, applying correction of the incidence angle effect using the Oren-Nayar model introduced by Carrea et al. (2016) and eliminating the atmospheric attenuation effect with the data-driven coefficients.

The proposed method was demonstrated on the data acquired in the Gouffre Georges cave in southern France, to distinguish two distinct rock types within the TLS point cloud. Part of the cave was formed on the lithological contact between marble and lherzolite, which name was derived from its type locality near Étang de Lers (previously Étang de Lherz). This ultramafic igneous rock was formed in the upper mantle and is rarely observable exposed on the surface (Sorriaux et al., 2019). Based on the obtained corrected intensity values, it is possible to identify and spatially locate individual types of mapped rocks, determine their dimensions and visualize them in a 3D fashion. Such results provide a combined view of underground and surficial geological settings which improves studying geological structures and understanding their formation. Along with conventional near-infrared spectroscopy the findings can become applicable in mining for underground mapping and mineral exploration (Laukamp et al., 2012; Guha et al., 2015), tunnel engineering (Cai et al., 2022), and planetary surface mapping (Horgan et al., 2014).

  - Background on laser scanning intensity

Laser scanning operates on the same physical principle as radar; therefore the radar equation can be applied to describe the main factors affecting the received signal strength (Wagner, 2010), providing a theoretical basis for the model-driven approach of the intensity correction. This equation relates to the received power Pr, transmitted pulse power Pt and other parameters referring to the acquisition geometry and sensor, atmospheric and target properties.

Pr=PtDr24πR4βt2ηsysηatmσ 			          (1)

Range R represents distance between the sensor and object, Dr is a diameter of the sensor receiver aperture, βt is the transmitter beamwidth, ηsys and ηatm are system and atmospheric transmission factors, respectively. All target parameters are combined into backscatter cross-section σ expressing the backscattering target characteristics as a function of the target size, reflectance, and scattering directionality (Wagner et al., 2006).

Assuming that the scanned area is an extended target larger than a laser footprint size and its surface has Lambertian scattering characteristics, the backscatter cross-section can be expressed as (Carrea et al., 2016; Xu et al., 2017):

σ=πρλR2βt2 cosα 					(2)

where ρλ is the target surface reflectance at a certain wavelength λ and α is the incidence angle, the angle between the surface normal vector and the incident laser beam.

Parameters Pt, Dr and ηsys depend on the sensor and can be considered constant for a specific device as well as the atmospheric attenuation ηatm for the TLS data acquisition under the stable conditions (Xu et al., 2017). After substituting Equation (2) into Equation (1) and simplifying for constants, the radar equation can be written as:

Pr=ρλR-2cosα  					(3)

The intensity value I represents the measure of the signal strength obtained by converting and amplifying the received power Pr, which is transformed into Digital Number (DN) as a scaled integer value (Höfle and Pfeifer, 2007; Tan et al., 2016) or provided in arbitrary units depending on the LiDAR device. As the intensity is generally proportional to the received power, then:

I ∝ Pr ∝ ρλR-2cosα     			   	    (4)

- Study site

The data lidar data subject to the presented research were acquired by laser scanning in the Gouffre Georges Cave, department of Ariège, southern France (Fig. 1). The cave formed in a specific geological setting of the Mesozoic carbonate cover of the northern Pyrenees , on the eastern slope of the Mont Béas, in the vicinity of the Étang de Lers.

The cave is unique for it is carved along the contact between two distinctive rock units: variably serpentinized lherzolites of mantle origin and marbles derived from Jurassic sedimentary carbonates. The degree of serpentinization of the peridotites varies greatly at the scale of the Lherz Massif and tends to increase near contacts, faults and shear zones. The unserpentinized lherzolite consists primarily of olivine (Fo89-Fo92), enstatite, Cr-diopside and Cr-spinel (Le Roux et al., 2007). The main serpentines include chrysotile and, to a minor extent, antigorite. The marbles are massive and banded monomictic to polymictic, locally dolomitic carbonate breccias. The most common minerals include calcite, dolomite, scapolite, phlogopite, and quartz. The dark bands in the marbles tend to be blueish due to the presence of scapolite and are continuous over several meters.

Fig. 1 Location of the Gouffre Georges cave. Selected parts for the TLS survey (red) of the Gouffre Georges cave complex (orange) is accessible via the entrance of Tube at the altitude of 1390 m. Base map: ©2021 Esri

The terrestrial laser scanning was performed in one branch of the cave corridors accessible via the Tube protected by a closable deck at the altitude of 1390 m. The narrow, 1 to 3 meters wide, entrance passage opens dips down and after about 70 meters of length enters a large dome of the Grande Galerie with over 10 to 15 m height, 20 m width and length of 130 m. The contact between marble and lherzolite is very well preserved in this dome. The dome continues down to the Salle de la Famine dome which formed in marble and marble breccia, but lherzolite debris was transferred from the upper parts forming the scree on the cave bottom. Overall, the surveyed part of the cave extended about 200 meters horizontally and 200 meters vertically with a general slope of the cave corridors of 45°. The conditions were extreme for surveying. The use of mountaineering equipment was required (Fig. 2). The scanner was carefully transported in a shock-proof metal case in a backpack. The air temperature was about 5°C and the humidity 98%, but the water content in the air is typically lower et the entrance and increases depending on the water source in the cave.

The scanned part of the cave provides a special opportunity to directly observe the rare geological and lithological setting. More than 40 lherzolite bodies crop out in the Pyrenean belt (Monchoux, 1970). These rocks have been exhumed during the Albian-Cenomanian rifting phase (e.g. Lagabrielle et al., 2010) and have been then uplifted and put in their present-day position during the Late Cretaceous-Tertiary Pyrenean orogeny (Verges et al., 2002; Saspiturry et al., 2020a). The North Pyrenean Zone, where such ultramafic rocks crop out, represents then the suture zone between the Iberian and the Eurasian plates. In the last decade, renewed interest has grown in the scientific community regarding the geologic history of the Pyrenean lherzolites (e.g., Lagabrielle & Bodinier, 2008; Jammes et al., 2009; Clerc & Lagabrielle, 2014). In particular, active debates have developed concerning the tectonic style related to the exhumation of mantle rocks during the Cretaceous rifting (e.g. Masini et al., 2014; Asti et al., 2019; Lagabrielle et al., 2019a, b; Saspiturry et al., 2019), its thermal implications (Clerc et al., 2015; Lagabrielle et al., 2016; Ducoux et al., 2019; Duretz et al., 2019; Saspiturry et al., 2020b), the kinematics of this extensional phase (e.g. Jammes et al., 2009; Tavani et al., 2018; Angrand et al., 2020) and the genesis of lherzolite-bearing breccia which is often found in association with the ultramafic bodies all along the North Pyrenean Zone (Lagabrielle & Bodinier, 2008; Debroas et al., 2010; Clerc et al., 2012; Saint Blanquat et al., 2016). The Lherz ultramafic massif is a key locality to find important constraints to all these scientific questions (see Clerc et al., 2012 and Lagabrielle et al., 2016 for exhaustive reviews).

Fig. 2. Scanning in the Gouffre Georges cave challenged the standard outdoor terrestrial laser scanning by a steep ground surface often with unstable scree or passing through narrow corridors. Careful transport in a shockproof metal case and use of mountaineering equipment was required.

  - Design of the intensity calibration experiment

Two sets of indoor experiments with different targets and varying acquisition geometry were conducted using the same scanner RIEGL VZ-1000. For the distance effect analysis, a white reference target mounted on a board was scanned for distances starting from 2.5 m with the next scanning position at the distance of 3 m from the device, and then each position with an increment of 1 m up to 60 m for the final position. The incidence angle was constant and close to 0° to eliminate its influence. The second experiment consisted of scanning rock samples collected during the cave survey. Each sample of marble and lherzolite were fixed on a revolvable base and scanned at the constant distance of 5 m and varying incidence angle incremented by 10° starting from 0° and approaching to 90° for the last scan. Lastly, each rock sample was scanned at a constant distance of 5 m and incidence angle of 0° in dry conditions and subsequently soaked for simulating the condensation in the cave environment and analysing its influence on the intensity data.

- Data and methods

The work undertaken in the presented research involved the workflow shown in Figure 3. The research commenced with the 3D mapping in the cave by TLS survey and processing of the data recorded. It was followed by scanning in a laboratory to calibrate the laser intensity recorded in the cave. Subsequently, the effects of the acquisition geometry and different surface conditions were analysed and the parameters required for the corrections steps were derived. Then the intensity correction for each influencing factor was performed taking the intensity calibration lab experiment into account. After correcting the intensity of each scan for the effect of distance and incidence angle, the derivation of the atmospheric attenuation coefficient was performed based on the data in overlaps of the individual scans. The last step involved denoising, i.e. removing the outliers by the nearest neighbour analysis and interpretation of the derived obtained corrected intensity values. Each step of the workflow is characterized in the following subsections.

Fig.3 Schematic workflow of the processing the data obtained by TLS survey and experimental scanning and subsequent parameter estimation for the intensity correction.

Alebo navrhujem vyhodiť odtiaľ obrázky/fotky, urobiť iba suchú textovu schému a obrázky použiť ako samostatné ďalej v texte.

  - Cave laser scanning and data processing

The selected part of the cave was scanned from 47 positions in total (Fig. 4) representing the entrance narrow passage of Tube and the domes of Salle de la Famine and Grande Galerie. Data were acquired by a long-range terrestrial laser scanner RIEGL VZ-1000 with 360° horizontal and 100° vertical field of view emitting near-infrared laser pulses laser of 1550 nm wavelength. The robust construction of the device enables scanning in adverse conditions and prevents its damage during transportation and abseiling in the cave. Besides high ranging precision and accuracy, implemented echo digitization and on-line waveform processing enables assigning additional attributes to each point measurement (Pfenningbauer and Ullrich, 2010).

The first 26 positions were captured with an angular step width set to Panorama 40 with an angular increment of 0.04° between two successive emitted laser beams. This ensured a high density of points and detailed scanning in those cave sections where terrain and wall geometry are complex and rock boundaries well detectable. The remaining 21 positions were located in the narrow passage from the Grande Galerie to the cave entrance of Tube. This was performed to geolocate the cave interior with the above-ground terrain surface and global geodetic coordinate system (i.e. global data registration). In these narrow parts, the scanning density of Panorama 60 with an angular increment of 0.06° was sufficient to record the high level of cave surface detail. The data of the individual 47 scanning positions from the cave comprised almost 656 million points in total. Subsequent mutual registration of the scanning positions in the RiSCAN PRO software by RIEGL using the iterative closest point adjustment of the consecutive scans termed Multi-Station Adjustment (MSA) (Ullrich ert al. 2003). The procedure minimises the 3D distance between groups of points identified as identical in the overlapping scans. Then it performs translation and rotation of the entire point cloud along X, Y, and Z axes until the least possible distance between the point groups is reached. Points captured within the range of 2.5 m from the scanner were removed from further processing. They were within the minimal scanning range for the device defined by the manufacturer with a high probability of positional inaccuracy. For obtaining the intensity values corresponding with the surface reflectance only, points recorded with the full emitted pulse captured as a single echo were used. The points recorded as multiple echoes or stray points with a high pulse shape deviation were filtered out.

The exterior of the cave was scanned from XX positions to capture the spatial context of the cave and the Mont Beas Mountain. Spatial transformation of the final point cloud to the French national coordinate system (EPSG code: XXX) was performed using XX control points measured with global navigation satellite positioning using the real-time kinematic method.

Fig.4 The top view and side view of the laser scanned part of Gouffre Georges comprising the domes of Salle de la Famine and Grande Galerie accessible via the Tube entrance with marked TLS positions (red dots) and cave exterior (green)

Premenuj v obrázku Front view na Side view

  - Intensity calibration experiment

The presented study also uses a TLS dataset acquired to calibrate the cave TLS point cloud for scanning distance (range) and laser incidence angle. Two sets of indoor experiments with different targets and varying acquisition geometry were conducted using the same scanner RIEGL VZ-1000 as in the Gouffre Georges. The effect of distance on the recorded laser intensity was approached by consecutive scanning a white reference target mounted on a panel stand starting from 2.5 m away from the scanner with the next scanning position at the distance of 3 m from the device, and then each position with an increment of 1 m up to 60 m for the last position of the white target (Fig. 3 – to,kde je schéma). The incidence angle with respect to the target surface was constant and close to 0° to eliminate its influence on this part of the calibration experiment. The second part of the experiment involved scanning two samples of rock collected from the cave during the TLS survey. Each sample of marble and lherzolite was fixed on a revolvable base on a tripod and scanned at a fixed distance of 5 m while varying the incidence angle. The samples were rotated along their vertical axis by a 10° increment starting from 0° with respect to the scanner approaching 90° for the last scan. Lastly, both pieces of rock were scanned from the 5 m distance and 0° incidence angle as dry samples. Afterward, the rocks were soaked in water for simulating condensation of air humidity on the rock surface in the cave environment and analysing its influence on the recorded laser intensity.

Before applying the intensity correction for each mentioned influencing factor on the Gouffre Georges dataset, the analyses of the effects of the acquisition geometry and different surface conditions, and deriving the parameters required for the corrections steps preceded. After correcting the distance and incidence angle effect of the whole dataset, the derivation of the atmospheric attenuation coefficient was performed based on the overlapping data. In the last step, the outliers were removed by the nearest neighbour analyses and obtained corrected intensity values were interpeted. Fig. 3 illustrates the complete workflow.

Fig.3 Schematic workflow of the processing the data obtained by TLS survey and experimental scanning and subsequent parameter estimation for the intensity correction

  - Intensity correction

The character of the caves, especially the complex cave walls geometry and variable dimensions of the cave spaces causing the near-distance effect can considerably influence the recorded intensity values. Moreover, a high concentration of particulates in the form of aerosol consisting of spores, dust, and water droplets is common in the cave environment and the relative humidity varies depending on the vicinity of the entrance. The relative humidity ranges a lot near the entrances, typically between 50 and 99 %. Deeper into the cave and further inside from the entrance, and where vadose seepage or flowing water is present, relative humidity is stable and close to 100% (James, 2013). Under these circumstances, the atmospheric attenuation factor should be taken into consideration when laser intensity is to be corrected.

For analysing the corrected intensity values Ic proportional to the object surface reflectance Ic∝f(ρλ), other influencing factors should be eliminated. According to Equation (4) effects of the main physical parameters affecting the value of intensity can be described as a function of the surface reflectance fρλ, distance effect f1(R), incidence angle f2α and for the presented research with the inclusion of the atmospheric attenuation effect f3(ηatm):

I=fρλ*f1R*f2α*f3(ηatm)   		          (5)

    - Correction of the distance effect

For the extended target, when the target is larger than the size of a laser footprint, so the entire energy of the laser footprint reflects on its surface, the backscattered signal power is inversely proportional to the squared range proceeding from the radar equation. However, the distance effect is dependent on the instrumental properties and does not follow the 1/R2 relation for near distances less than 10 - 15 m varying for different laser scanners (Kaasalainen et al., 2011). The explanation for the near-distance effect links to the device equipped with the amplifiers for low-reflectance and brightness reducer for near distances. As the continuous function in a closed interval can be approximated by the polynomial series, the range function for the particular device can be data-driven from the reference measurements (Tan et al., 2016). Therefore, the function of the distance effect f1R for a specific instrument can be expressed as:

f1R=k=0n pkRk  		  		 	(6)

where pk stands for the polynomial coefficients, which were estimated from the measured intensity of the reference target of the calibration experiment using least-squares adjustment.

Vzal som odtiaľ celú sekciu opisujúcu výsledok a dal do results (Corrected intensity)

    - Correction of the incidence angle effect

The incidence angle enclosed by the surface normal vector and the incident laser beam can be calculated from the measured point coordinates and the scanner position. Assuming the surface has perfect diffuse scattering characteristics (i.e. Lambertian surface), the backscattered intensity is directly proportional to the cosine of the incidence angle according to Lambert´s cosine law (Xu et al., 2017). With increasing irregularities of the natural surfaces, the deviation from the Lambertian model increases. These surfaces can be modelled using Oren-Nayar model based on the bidirectional reflectance distribution function (Oren and Nayar 1995, 1994). The geometry of the natural surface is described as a set of micro-facets with different orientations, returning more light in the direction of the light source more than the surface following the Lambertian reflectance model. Carrea et al. (2016) implemented the Oren-Nayar reflectance model into the correction of the incidence angle effect taking into account the microstructures as the surface roughness. As in TLS the incident and reflected laser beams are coincident, function using the Oren-Nayar model can be simplified as follows:

f2α= cos(α )A+B sin(α) tan(α )	 		   (7)

A=1-0,5σslope2 σslope2 +0.33        B=0,45σslope2 σslope2 +0.09

The parameter σslope represents the standard deviation of slope angle distribution of the micro-facets. As σslope depends on the surface roughness only, it was estimated from the reference dataset of different incidence angles previously corrected for the distance effect therefore the intensities were dependent on the surface properties only. Since the scanned surface contained many irregularities causing deviations of the incidence angle, only sample points with the integer values starting from 0° with a 10° increment up to 80° and the last dataset containing points with a maximum incidence angle of 89° were selected for the σslope parameter estimation. Sampled dataset of the selected points was then repeatedly corrected for the incidence angle effect according to Equation (7) with the value of σslope  set from 0° to 90° with an increment interval of 1°. The σslope value was estimated as the optimal value minimizing the variation of corrected intensity of all incidence angles.

    - Correction of the atmospheric attenuation effect

The atmosphere causes the diminution of the laser pulse while propagating through the atmosphere. It is related to the average atmospheric conditions during scanning, e.g. humidity, ambient temperature, aerosol concentration, and it can be described as (Höfle and Pfeifer, 2007; Ding et al., 2013):

f3(ηatm )= 10-2Ra/10000    		 	         (8)

where a is the atmospheric attenuation coefficient. The attenuation of laser power for the horizontal propagation can range from 0.2 dB/km for clear conditions, 3.9 dB/km for haze conditions (Jelalian, 1992) up to more than 300 dB/km in very thick fog (Kim et al., 2001). For modelling the atmospheric effect, the input detailed meteorological data or corresponding visibility are required. Such atmospheric measurements are not usually done in caves or obtaining such data in an irregular cave complex is difficult or even impossible, therefore an approximated value has to be defined. Assuming that differences in intensity values of the selected data with the same target properties are related to the atmospheric attenuation function, the attenuation coefficient a was determined for the specific cave parts from the overlapping data of different scanning positions after correcting the data for distance (range) and incidence angle.

  - Denoising

After the intensity correction, all scanning positions were merged into a single point cloud and again spatially homogenized with a regular point spacing of 5 mm to remove duplicate points in the overlapping parts of several scanning positions. The corrected intensity of the final point cloud was smoothed and outliers of extreme intensity were removed to obtain the most satisfactory results for intensity interpretation. For this task, the average intensity value of the 10, 25, 50, and 100 nearest surrounding points was successively tested in the nearest neighbour analysis. Denoising and visualization were performed in the CloudCompare software (Girardeau-Montaut, 2018).

  - Point cloud classification based on the corrected laser intensity

The corrected intensity of the denoised final point cloud was used to differentiate the lithology apparent on the cave surface. For this task, the points were classified using threshold values of corrected intensity based on the laser scanning of rock samples in the lab calibration experiment. We used one standard deviation with respect to the mean value of corrected intensity for dry and wet conditions to classify the points as marble or lherzolite.

The accuracy of the point cloud classification was assessed by comparing the coloured intensity with the photographs taken in the Grand Galerie dome. The photographs were spatially aligned to the TLS point cloud after the laser intensity correction in the Meshlab software (Cignoni et al., 2008) using mutual point correspondences. The points were assigned with the natural colour as RGB triplets and the outlines of the lherzolite and marble rock were identified by visual interpretation of the natural colour. Thus each point was classified as marble or lherzolite based on the corrected intensity and natural colour. The classification accuracy was assessed using a confusion matrix (Tab. 1). True positives (TP) refer to all coincidently identified marble points and true negative (TN) to all coincidently identified lherzolite points according to the RGB and corrected intensity values. False positives (FP) stand for all marble points falsely identified as lherzolite and false negatives (FN) denote all lherzolite points falsely identified as marble.

Tab. 1 Confusion matrix used for the classification accuracy assessment.

|  |  | Reference |  |
| --- | --- | --- | --- |
|  |  | Marble | Lherzolite |
| Predicted | Marble | TP | FP |
|  | Lherzolite | FN | TN |

To evaluate the spatial correspondence between the classification based on the referenced RGB values and corrected intensity values representing different rock types, three indices were calculated, including marble accuracy (recall), lherzolite accuracy (specificity) and overall accuracy defined as follows:

Marble accuracy=TPTP+FN

Lherzolite accuracy=TNTN+FP

Overall Aaccuracy= (TP+TN)(TP+FP+FN+TN)

- Results

Laser scanning in the Gouffre Georges cave resulted into its first and highly detailed 3D digital representation applicable in geoscience research or in planning and protection of this unique environment. The acquisition and processing of the 47 individually scanned point clouds in the cave generated the final data set comprising over 218.3 million of points with regular spacing set to 5 mm at a 10-meter range. The mutual orientation of the scanning positions achieved 6.7 mm of standard deviation demonstrating a very high level of overall registration accuracy. The final data provided the first highly detailed 3D perception of the cave and enabled the calculation of the scanned volume (44,450 m3) and scanned surface (23,808 m2).

To communicate the scientific results to a wide community, we generated a stand-alone online web portal (Fig. 5) providing other researchers, professionals, or public means for exploring the dataset without a need for any special skills in computer graphics, geoinformation science, installation of a special kind of software, or any plugins. The visualization tool is based on the Potree open-source WebGL renderer (Scheiblauer et al. 2014; Schuetz, 2016; Potree, 2021) and it was generated using the laspublish software utility of the LAStools package (Rapidlasso GmbH, 2019). The user can efficiently visualize the millions of lidar points in real-time via the Internet, change the colour based on point attributes, measure vertical or slant distances, calculate area and volume, generate vertical profiles, and export the data. The presented dataset can be accessed via the link in the caption of Figure 5.

Fig. 5. The interface of the web portal for interactive viewing and analysis of the final TLS point cloud shows various point attributes including the corrected laser intensity, and terrain surface. Link for access: https://uge-share.science.upjs.sk/webshared/Laspublish/GOUFFRE_GEORGES/GouffreGeorges.html

Doplnte do las publish atribút pre body z klasifikácie (lherzolit zeleny, mramor zlty, voda sivomodra, idealne iba 2 triedy, ak sa to da

  - Corrected laser intensity

The final and unified single pointcloud contains the corrected intensity values which resulted from applying the approach developed in this this study. The originally recorded laser intensity was corrected individually for each of the 47 scans. First, the intensity data were corrected for the influence of the distance (scanning range) based on the polynomial functions derived from the calibration lab experiment (Section 3.2, 3.3) using Equation 6. Fig. 5 – experimental/model curve below).. The coefficients of the equation are reported in Table XX (uveď dole)

Figure 5 demonstrates how the laser intensity recorded by RIEGL VZ-1000 markedly deviates from the radar range equation (Fig. 5 – treba tam doplniť krivku radar equation). Starting from the 2.5 m, declared as the minimum scanning range, intensity first rapidly decreases up to 4 m and then increases as a result of instrumental effect for the near distances. The peak is observed at the distance of around 10 m and then the recorded intensity values are considerably attenuated with the increasing range, approaching the radar equation function. To achieve a higher accuracy of fitting the model curve to the empirical data, the ranges were divided into three intervals: below 5 m, 5 - 15 m, above 15 m, and the polynomial coefficients were derived according to Equation (6). Doplniť info o odchýlkach polynomického modelu a empirických meraní (v R sa to dá, v Exceli neviem)

Fig. 5 Intensity of the white targets as the function of scanning range recorded by the RIEGL VZ-1000 scanner in the lab experiment. Fitting of the empirical polynomial curve is based on the radar equation. Minimum scanning range and separation ranges of polynomial functions are marked with semi-dashed and dashed vertical lines, respectively. The theoretical curve of the radar equations is shown in black. Doplň sem krivku teoretického modelu pre lepšiu názornosť celého obrázku, lebo píšeš, že experimentálna krivka nesleduje radarovú krivku, ale nie je to bez nej jasne vidieť. Zmeň názov osi X ako “Scanning range”, bude to jasnejšie, o čo ide.

Uviesť samostatne graf a tabuľku z obrázku. Nedá sa na ňu takto ani odvolať, a je dosť podstatná pre vysvetlenie výsledkov, taktiež v legende uveď namiesto “reference targets” = “measured intensity”, polynomial function = “fitted polynomial model”.

Determination of the σslope parameter for each material improved the intensity correction results, however, the implementation is unfeasible without prior material differentiation within the entire dataset. – nerozumiem, napíš po slovensky. Even a single optimized parameter significantly reduces the overcorrection of the high incidence effect for the rock surfaces with non-Lambertian behaviour (Carrea et al., 2016). Therefore a single σslope  value of 29° was determined to be applicable for the entire Gouffre Georges data set minimizing the incidence angle effect.

While the adjustment of intensity for the effect of scanning range and incidence angle improved the perception of laser intensity the result was still not entirely satisfactory. It was observed in the overlapping parts of the scans. Table XX and Figure 6 demonstrate this problem on a representative dataset from the dome of Salle de la Famine with walls comprising marble only. The wall was scanned from three positions, i.e. ranges. The three curves of raw intensity distribution do not match (Fig. 6A) and the highest intensity occurs at the shortest range and vice-versa. After the corrections for distance and incidence angle, the differences between the distribution curves are reduced although not perfectly (Fig. 6B). The range dependence is still apparent. The reason is the energy loss due to the atmospheric attenuation of the laser pulse. Therefore, the intensity was further corrected using Equation (8) to prevent the confusion of the effects of atmospheric conditions and differing surface material (Fig. 6C). In the particular scanned cave environment, the coefficient of atmospheric attenuation a varied from 0.7 – 1.8 dB for the maximal scanning ranges of 60 m in the selected overlaps of scans, depending on the distance to the cave entrance or to the water flow. Toto je cenný výsledok a by bolo dobré preukázať ho na hodnotách napr grafom alebo tabuľkou, kde bude na osi X vzdialenosť od vchodu a na osi x intezita, tri rôzne ranges pri ktorých si určovala intenzitu budú rôznou farbou ako 3 paličky, farba ako v grafe 6.

Table XX: Summary statistics of the intensity distribution (in dB) of the overlapping data acquired from three scanning positions, i.e. three different ranges (41.1 m, 32.9 m, 21.5 m) in the selected area of the dome of Salle de la Famine: (A) raw intensities, (B) intensities after the correction for distance and incidence angle, (C) final intensity values after the correction for the atmospheric attenuation. The low index values refer to the scanning ranges as in Figure 6.

|  | A41.1 | A32.9 | A21.5 |  | B41.1 | B32.9 | B21.5 |  | C41.1 | C32.9 | C21.5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Minimum | 22.360 | 24.240 | 26.850 |  | -9.070 | -8.303 | -7.887 |  | -5.920 | -5.813 | -5.999 |
| Maximum | 29.860 | 32.200 | 35.670 |  | -1.880 | -0.364 | -0.162 |  | 1.143 | 2.128 | 1.648 |
| Range | 7.500 | 7.960 | 8.820 |  | 7.190 | 7.939 | 7.726 |  | 7.063 | 7.940 | 7.647 |
| Mean | 26.660 | 28.450 | 31.970 |  | -4.782 | -4.163 | -3.556 |  | -1.681 | -1.679 | -1.743 |
| Std | 0.832 | 0.929 | 1.175 |  | 0.777 | 0.978 | 1.163 |  | 0.771 | 0.975 | 1.158 |
| 1st quartile | 26.180 | 27.900 | 31.280 |  | -5.234 | -4.756 | -4.277 |  | -2.128 | -2.272 | -2.468 |
| Median | 26.740 | 28.510 | 32.030 |  | -4.718 | -4.075 | -3.459 |  | -1.616 | -1.591 | -1.642 |
| 3rd quartile | 27.230 | 29.120 | 32.780 |  | -4.255 | -3.481 | -2.725 |  | -1.154 | -0.996 | -0.920 |

Fig. 6 intensity distribution of the overlapping data acquired from three scanning positions, i.e. three different ranges (41.1 m, 32.9 m, 21.5 m) in the selected area of the dome of Salle de la Famine: (A) raw intensities, (B) intensities after the correction for distance and incidence angle, (C) final intensity values after the correction for the atmospheric attenuation.

The range dependence of the original intensity values is most evident when combining the individual scanning positions (Fig. 7A). Data shadows occurred primarily in the most constricted parts and also in vertical transitions, where stabilization of the scanner was dangerous. In such parts, no 3D surface model was interpolated from the TLS data. It would interpolate points even in the areas of evident data shadows and distort the resulting interpretation of real recorded intensity values and the mapped objects. In the narrow passage leading from the Tube entrance down to the Grande Gallerie dome, the data scanned closer to the scanner than the recommended minimum scanning range of 2.5 m were kept for visualization purposes. In these confined cave sections, the TLS captured points at a distance of ca. 0.9 m, but with a certain position inaccuracy. The further results on the interpretation of the corrected intensity data concern the large domes of Grande Galerie and Salle de la Famine.

Vieme tu uviesť ako sa správala intenzita so vzdialenosťou medzi 0.9 – 2,5 m? Celkom by to bolo zaujímavé zobraziť nap raj v obr. 5, alebo osobitne, aby bolo vidieť, aký rozptyl alebo nespoľahlivosť/nesystematickosť má intenzita pri blízkych vzdialenostiach, ale chápem, že tu ide aj o nepresnosť merania vzdialenosti.

The original intensity data of each scanning position rapidly decreases with the increasing range of scanning (Fig. 7A). Therefore, the highest intensity values are primarily in the center of the scanned point cloud. Merging these datasets into a single point cloud without any correction would lead to the misinterpretation of the surface materials. After eliminating the distance effect, influencing factors related to incidence angle (i.e. surface geometry), and the atmospheric attenuation effect, the resulting corrected intensity values depend primarily on the surface material. Even though data before applying the correction indicate different materials, acquired corrected intensity values delimited material borders and sections with water occurrence and the watercourse in the lower part of the Grande Galerie dome. Areas in the vicinity of the watercourse evidence the lowest intensity values as a consequence of the water droplets. Similarly, the histogram distribution of the corrected intensity indicates the presence of three main different types of surfaces.

Fig.7 Final point cloud comprising of 47 scanning positions coloured by the A) range values B) original intensity C) corrected intensity. Doplň A , B, C ku obrázakom, aby sa dalo na ne osobitne odvolávať a bolo konzistenté naprieč článkom

By applying the denoising process of nearest neighbour analysis, outliers comprising 0.015% points were removed from the entire dataset. Due to the high density of the recorded data, it performs to be more appropriate to use a higher number of 100 neighbours, significantly smoothing the data and reduce noise points. After this denoising step, the histogram distribution is also more delimited representing intensity limits for the different types of the surface (Fig 8). Although smoothing may increase the uncertainty of the boundary position between the observed rocks by its shifting in the direction of the prevailing values, it allows better distinguishing of the significant areas of different rock types contact. One of these most noticeable features, the significant contact of lherzolite and marble is visible in the upper part of the Grande Galerie dome. The indicator of this strong contrast was visually recognizable from the original intensity values. However, the influence of the distance effect is apparent resulting in intensity values decrease with increasing range for the displayed scanning positions from ranges of about 20, 40 and 55 metres described within the upper right graph in Fig. 8. After the intensity correction, the boundaries are clearly demarcated the overlapping data of different ranges evince similar intensity values and even the distribution indicates two distinct materials in the lower right graph.

Fig.8 Top and side view of the selected Gouffre Georges cave parts point cloud with corrected intensity values after removing the outliers by the denoising, revealing the lithological contacts and the water occurrence. In the top right picture (photograph), dark-green rock is lherzolite and white-pinkish rock is marble. Note the remarkable fitting between the lithological differences and the intensity data, before and after correction.

Premenuj v obrázku Front view na Side view plus dopln grafickú mierku v osi Z do Side view

  - Lithological differentiation based on the corrected laser intesity

The primary lithological differentiation of the lidar points was based on the simple classification approach described in Section 3.5. The exploratory statistical analysis indicated that distinguishing between dry and wet rock surface is important. Figure 9 demonstrates this effect as a result of scanning rock samples in the lab experiment by the same Riegl VZ-1000 device as in the cave. First, the recorded intensity values were corrected for scanning range and atmospheric attenuation using the equations XX. The mean and standard deviation of the distribution of corrected intensity were obtained for dry and wet conditions. Moistening the surfaces caused a shift towards lower intensity values in both cases. The intensity values ​​of lherzolite decreased on average by 3 [dB] and marble by 1.8 [dB], while the decrease in the surface reflectivity represents almost 13% for lherzolite and 22% for marble as a result of water absorption of the near infrared wavelengths. Different rate of decline is related to the amount of applied water, the rock properties and its ability to absorb the water. As the two rock types still remained distinguishable for both wet and dry conditions, we assumed the corrected intensity values allow for differentiating the lherzolite and marble rock even in the cave environment with presumable thin water layer condensed on the walls.

Fig. 9 Distribution of the corrected intensities for the lherzolite and marble samples observed under the dry and wet surface conditions. Označ grafy A-D a uveď v popise obrázku čo A-D reprezentuje, mne to nie je jasné o akú korkeciu išlo (scanning range a atmosphere? Hodnoty sample mean a stdev v rámčeku nie je dobre vidieť, štatistiky uveď tabuľkou podpbne ako pre figure 6

The classification of the points was performed by two approaches and the results are presented in the example of the upper part of Grande Galerie captured also in a photograph (Fig. 8, 10A). The first classification was based on the thresholds defined from the corrected intensities (Fig. 10B) of the rock samples (Fig. 10C), thus derived in the lab experiment. The other classification was based on the thresholds defined from the histogram of the corrected intensity recorded of the entire final point cloud (Fig. 10D), thus derived from recordings from the natural cave environment. The threshold intensities ​​for classifying the lidar points into the two rock classes were determined independently of the state of the surface wetness for each scanning position. The intensity limit was set from -10.038 to -4.014 dB and -4.551 to -1.132 dB, for lherzolite and marble respectively. The points with intensities below the lower threshold of lherzolite (-10.038) were considered as strongly influenced by the presence of water as their intensity values were similar to the values in the lower part of Grand Galerie with the watercourse. Hence, the points were assigned to the class of water (Fig. 10C). With the slight overlap of the lower-intensity threshold for the wet marble and the upper-intensity threshold of the dry lherzolite (Fig. 9C, 9D), a misinterpretation of the surface materials is expected for 5.14% of points from the entire dataset. Therefore, data in the interval of -4.551 to -4.014 [dB] were omitted for the classification assessment (tu myslíš accuracy assessment?, ja by som ich nevylučoval z hodnotenia presnosti, lebo ohýbame výsledky, či?).

The second classification approach used the intensity limits determined from the histogram distribution of the corrected intensity values after removing the outliers from the final point cloud (Section 3.4, Fig. 8). The threshold of -4.783 [dB] was used to distinguish lherzolite and marble and 12.207 [dB] was applied to separate lherzolite from water.

The results of the material differentiation are presented on the lithological contact in the Grande Galerie dome (Fig. 10).  The assessment of the classification accuracy was performed on the selected point cloud shown in Fig. 10. The data contained over 1.896 million points with assigned RGB values and three classes determined in the lab and the cave environment. As the water occurrence is not recognizable from the RGB information, the points representing this class identified by both approaches were not included in the assessment. This group comprised 108,380 points (5.71 %) based on the first approach and 5,105 points (0.26 %) based on the second classification approach. Tab. XX2 reports the calculated classification accuracy with the measures defined in Section 3.5. For both approaches, the overall accuracy is high (over 86 %) and mapping marble is more accurate than mapping the lherzolite points. Higher overall accuracy was achieved using the second approach in which the threshold intensities came from scanning the cave. The reason for the lower accuracy of the first approach based on the lab recorded intensity of rock samples is likely in the limited information contained by the sample rocks as opposed to the intensity recorded in the final dataset covering the surface of the entire cave under natural conditions.

Fig. 10 Section of the 3D TLS point cloud from Salle de la Famine (Fig. 7) comprising a cave wall coloured (A) by photographed natural colours with dark lherzolite and light marble, (B) by the corrected intensity values, (C) by the result of classification based on the thresholds from corrected intensities of the rock samples, and (D) by the result of classification based on the thresholds from the histogram of corrected intensity of the cave.

Tab. XX2 Classification accuracy of the two approaches based on the lab-recorded intensity (Classification 1) and cave-recorded intensity (Classification 2).

|  | Marble Accuracy | Lherzolite Accuracy | Overall Accuracy |
| --- | --- | --- | --- |
| Classification 1 | 97.96 % | 80.90 % | 86.18 % |
| Classification 2 | 95.19 % | 85.47 % | 89.24 % |

Doplnil by som tabuľku kde budú uvedené počty bodov (všetky body, body mramor, body lherzolite, vody, body withdrawn from the classification, body unclassified ak sú?)

- Discussion

Our results show that TLS intensity data can be used to distinguish the rock material of the scanned cave surface. Different intensity values between rock types were related not only to the reflective properties of the surface material but also to the system effects from different TLS settings and environmental conditions. The analyses of laser intensities from the natural cave and lab environments under two kinds of rock surface wetness conditions indicate that intensity data can be used to differentiate the two rock types. Thus, scanning the cave provided a highly detailed perception of the 3D geometry and also the properties of the rock surface.

This outcome cannot be efficiently achieved by passive remote sensing requiring a powerful light source to capture photography of high resolution and contrast in such a large and dark space.

Figure XX5 demonstrates the key benefit of laser scanning in the caves where the darkness of the environment refrains from taking standard photography for visual inspection or even applying close-range photogrammetric approaches to model the 3D cave geometry.

Since it was recently proven, also digital photogrammetry represents a beneficial method for acquiring 3D cave datasets for analysis of morphologies and geological structures, using a digital camera for ground-captured photographs (De Waele et al., 2018) or combining with an unpiloted aerial vehicle (UAV) (Triantafyllou et al., 2019). As the main limiting factor is related to the limited light conditions, artificial illumination is a necessity, e.g. in form of a flashlight on the camera or spotlights. However, the irregular shapes and complex wall geometries may cause shadows and overexposure, resulting in low-quality model reconstruction and the exposure variation can lead to the misinterpretation in material differentiation and can be problematic for further analysis and lithology interpretation. Ensuring optimum illumination for the Gouffre Georges spaces of such variable dimensions with abseiling parts is hardly possible. For the demonstration purposes at the part of the Salle de la Famine, where the dome dimensions were not so extensive thus the flash of the scanner external camera was partly sufficient to illuminate the surroundings, a scanning position was supplemented with the scanner camera photos. After the acquired points were coloured based on the RGB, the inconsistent light conditions appeared at the overlaps as well as the presence of the shadows (Fig 5). Using the corrected intensity values to visualize the selected part, particular objects are recognizable even in the distant “shadow” part and different materials are distinguishable. The lowest values correspond to the lherzolite bottom debris, with occasional fallen marble occurrence of higher intensity values. Walls composed of marble show higher intensities in general.

Recent studies demonstrated the applicability of digital photogrammetry for acquiring 3D cave datasets for analysis of morphologies and geological structures, using a digital camera for ground-captured photographs (De Waele et al., 2018) or combined with an unpiloted aerial vehicle (UAV) (Triantafyllou et al., 2019). Also, a combination of digital photography and laser scanning was successfully used in a cave to reveal painted black rock art behind graffiti and moss (Jalandoni et al., 2021). In these studies, the distance from the camera to the photographed surface defines the quality of the captured images. The light conditions in the cave are the main limiting factor of applying passive remote sensing. Artificial illumination must be used either in the form of a flashlight on the camera or spotlights in the scene. However, these devices usually do not provide sufficient and continuous light power to illuminate the surface at far distances of several tens of meters or their application is not feasible to map large cave spaces. Also, the irregularity and complexness of cave morphology introduce shadows and overexposure, resulting in low-quality 3D model reconstruction. The exposure variation can lead to the incorrect differentiation of material materials and can be problematic for further analysis and interpretation of lithology.

The extent of cave space in Gouffre Georges was in the order of tens and hundreds of meters which is too large to ensure optimum illumination by contemporary lighting technologies. In the abseiling parts, the use of photography is hardly possible.

For the demonstration purposes at the part of the Salle de la Famine, where the dome dimensions were not so extensive thus the flash of the scanner external camera was partly sufficient to illuminate the surroundings, a scanning position was supplemented with the scanner camera photos. After the acquired points were coloured based on the RGB, the inconsistent light conditions appeared at the overlaps as well as the presence of the shadows (Fig 5). Using the corrected intensity values to visualize the selected part, particular objects are recognizable even in the distant “shadow” part and different materials are distinguishable. The lowest values correspond to the lherzolite bottom debris, with occasional fallen marble occurrence of higher intensity values. Walls composed of marble show higher intensities in general.

Fig. XX5 Photography (A) in the dark cave environment is problematic for achieving sufficient light conditions while active laser scanning provides an intensity image (B) applicable for mapping the surface material. Example of the same view location in the Salle de la Famine in the Gouffre Georges cave.

Alebo nieco na tento styl (mozes dat farebnu tu foto intensity ako bola povodne plus pribliznu graf mierku .

Classification of the point cloud to map lherzolite and marble based on the corrected intensity has some limitations. The experimentally scanned collected rock samples do not represent a wide variability of the intensity values ​​for the entire cave environment. Despite this fact, when selecting data based on the defined limits, it is possible to effectively distinguish individual types of materials compared to the photo evidence. Subthreshold values ​​appearing as water can be attributed to lower values ​​for lherzolite, which can be included in the selection based on histogram distribution, better capturing individual material contrasts.

The presence of water considerably affects the discrimination of the mapped rock types as the moisture content causes a shift toward lower intensity values. The water occurrence as a thin water layer covering the rocks in the close vicinity of the watercourse complicates the material differentiation based on the corrected intensity in the lower part of the Grande Galerie. The presence of water on the scanned surface causes a shift toward the lower intensity values. Consequently, in these parts of the cave, the determined threshold values ​​of corrected intensity for marble can be assigned as dry lherzolite or water. This fact indicates the limitation of using of the corrected intensity for the material differentiation in the water stream surroundings or places of water seepage characteristic for the cave environment. Therefore, it is necessary to map and interpret the wet parts of the cave individually and use supplementing data sources and field measurements.

The results of the lidar survey in the Gouffre Georges cave show that intensity data, albeit a by-product of the survey and acquired 3D geometry, provide valuable information for rapid and detailed mapping of rock types in underground environments.

- Conclusion

The caves are typically a dark and humid environment limiting the use of mapping technologies based on passive remote sensing or the human eye. The benefit of active remote sensing by laser scanning in such conditions was demonstrated by a plethora of studies. While most of them exploit the high accuracy of capturing 3D geometry, few focus on the use of the laser intensity which is simultaneously recorded with the 3D point position.

The presented case study in Gouffre Georges cave demonstrated an efficient and complex intensity correction procedure for data acquired in the caves for mapping cave surface material. The key step in this approach is in the elimination of the influencing of factors related to the scanning geometry, surface roughness, surface spectral properties, and also atmospheric conditions. The experimental results demonstrated the significant effect of short scanning range which is unique for the deployed type of the terrestrial laser scanner. The presence of water, i.e. wet surface, introduced bias of the intensity values towards lower values complicating the separability between wet marble and dry lherzolite. Such conditions have to be considered applications of the recorded laser intensity in mapping.

The geological and lithological setting of the Gouffre Georges is rather unique in the whole Pyrenean belt since it allows to observe with a spectacular continuity and without any vegetal cover the contact between marble and lherzolite and tectonic structures that characterize such contact. These characteristics allow finding important pinpoints for geological questions still open in the region, such as the kinematics of structures related to the Cretaceous phase, the nature of the contact between marble and lherzolite, and the geologic significance of the marble or lherzolite bearing breccia. In this context, the laser scanning of the cave geometry assigned with the corrected laser intensity is an invaluable tool to unravel the complexity of such a lithological environment. This technique also allows putting geological observations in an absolute spatial reference frame, which is often very difficult in a cave environment.

This work contributes to the growing body of studies demonstrating the potential of lidar remote sensing even in the underground environment by showing that it has the potential to differentiate rock material using laser intensity data.

Future directions would focus on mapping the lithology of wet cave surfaces and the presence of more than two rock types using a different laser scanner to control for system effects. Another aspect of the next research could be in testing the classification results generated by sophisticated methods of machine learning, such as random forests or computational neural networks. We used a simplified radar equation under the assumption that the target was an extended Lambertian surface. The effect of incidence angle was corrected by using Lambert’s cosines law. In future research, we will consider a realistic illumination model by taking into account the superposition of specularly reflected light, diffuse reflected light, and ambient light to eliminate the incident effect and improve the actual precision of intensity values. Overall, this will lead to more reliable classification results with improved differentiation between rock types.

The geological and lithological setting of the Gouffre Georges is rather unique in the whole Pyrenean belt, since it allows to observe with a spectacular continuity and without any vegetal cover the contact between marble and lherzolite and tectonic structures that characterize such contact. These characteristics allow to find important pinpoints for geological questions still open in the region, such as the kinematics of structures related to the Cretaceous phase, the nature of the contact between marble and lherzolite and the geologic significance of the marble/lherzolite bearing breccia. In this context, the laser scanning of the cave and the correction of intensity data are an invaluable tool to unravel the complexity on such lithological environment. This technique also allows to put geological observations in an absolute spatial reference frame, which is often very difficult in a cave environment.

Acknowledgement

Please include acknowledgements to other grants /funding bodies as you need

The presented research originated thanks to the financial support of the Ministry of Education, Science, Research and Sport of the Slovak Republic under the grant nr. VEGA 1/0300/19 and VEGA 1/0798/20.

RA wish to acknowledge the Orogen Project (BRGM, Total, CNRS-INSU) for providing a post-doc grant. Yves Lagabrielle is warmly thanked for helping the scientific organization of the expedition in the cave.

Robert Guinot and the SCHS (Spéléo Club du Haut Sabarthez) speleological club of Tarascon-sur-Ariège are greatly thanked for their support during the acquisition days.

References

Gallay, M., Kaňuk, J., Hochmuth, Z., Meneely, J. D., Hofierka, J., & Sedlák, V. (2015). Large-scale and high-resolution 3-D cave mapping by terrestrial laser scanning: a case study of the Domica Cave, Slovakia. International Journal of Speleology, 44(3), 6.

Idrees, M. O., & Pradhan, B. (2016a). A decade of modern cave surveying with terrestrial laser scanning: A review of sensors, method and application development. International Journal of Speleology, 45(1), 8

Triantafyllou, A., Watlet, A., Le Mouélic, S., Camelbeeck, T., Civet, F., Kaufmann, O., ... & Vandycke, S. (2019). 3-D digital outcrop model for analysis of brittle deformation and lithological mapping (Lorette cave, Belgium). Journal of Structural Geology, 120, 55-66.

Burens, A., Grussenmeyer, P., Guillemin, S., Carozza, L., Lévêque, F., and Mathé, V.: Methodological developments in 3D scanning and modelling of archaeological French heritage site: the Bronze Age painted cave of "Les Fraux", Dordogne (France), Int. Arch. Photogramm. Remote Sens. Spatial Inf. Sci., XL-5/W2, 131–135.

Kruger, A., Randolph-Quinney, P., & Elliott, M. (2016). Multimodal spatial mapping and visualisation of Dinaledi Chamber and Rising Star Cave. South African journal of science, 112(5-6), 1-11.

Landeschi, G., Apel, J., Lundström, V., Storå, J., Lindgren, S., & Dell’Unto, N. (2019). Re-enacting the sequence: combined digital methods to study a prehistoric cave. Archaeological and anthropological sciences, 11(6), 2805-2819.

Silvestre, I., Rodrigues, J. I., Figueiredo, M., & Veiga-Pires, C. (2015). High-resolution digital 3D models of Algar do Penico Chamber: limitations, challenges, and potential. International Journal of Speleology, 44(1), 25-35.

Gallay, M., Hochmuth, Z., Kaňuk, J., & Hofierka, J. (2016). Geomorphometric analysis of cave ceiling channels mapped with 3-D terrestrial laser scanning. Hydrology and Earth System Sciences, 20(5), 1827.

Fabbri, S., Sauro, F., Santagata, T., Rossi, G., & De Waele, J. (2017). High-resolution 3-D mapping using terrestrial laser scanning as a tool for geomorphological and speleogenetical studies in caves: An example from the Lessini mountains (North Italy). Geomorphology, 280, 16-29.

De Waele, J., Fabbri, S., Santagata, T., Chiarini, V., Columbu, A., & Pisani, L. (2018). Geomorphological and speleogenetical observations using terrestrial laser scanning and 3D photogrammetry in a gypsum cave (Emilia Romagna, N. Italy). Geomorphology, 319, 47-61.

Santos Delgado, G., Martínez Rubio, J., Silva Barroso, P. G., Sánchez Moral, S., Cañaveras Jiménez, J. C., & De la Rasilla Vives, M. (2010). Contribución al conocimiento de la cueva de El Sidrón (Piloña, Asturias) con técnicas de láser escáner 3D. Avances de la Geomorfología en España, 2012, 255-258.

Citton, P., Romano, M., Salvador, I., & Avanzini, M. (2017). Reviewing the upper Pleistocene human footprints from the ‘Sala dei Misteri’in the Grotta della Bàsura (Toirano, northern Italy) cave: An integrated morphometric and morpho-classificatory approach. Quaternary Science Reviews, 169, 50-64.

Azmy, S. N., Sah, S. A. M., Shafie, N. J., Ariffin, A., Majid, Z., Ismail, M. N. A., & Shamsir, M. S. (2012). Counting in the dark: non-intrusive laser scanning for population counting and identifying roosting bats. Scientific Reports, 2, 524.

McFarlane, D. A., Roberts, W., Buchroithner, M., Van Rentergem, G., Lundberg, J., & Hautz, S. (2015). Terrestrial LiDAR-based automated counting of swiftlet nests in the caves of Gomantong, Sabah, Borneo. International Journal of Speleology, 44(2), 6.

Milius, J., & Petters, C. (2012, July). Eisriesenwelt–From laser scanning to photo-realistic 3D model of the biggest ice cave on Earth. In GI-Forum (pp. 513-523).

Berenguer-Sempere, F., Gómez-Lende, M., Seranno, E., & de Sanjosé-Blasco, J. J. (2014). Orthothermographies and 3D modeling as potential tools in ice caves studies: the Peña Castil Ice Cave (Picos de Europa, Northern Spain). International Journal of Speleology, 43(1), 4.

Šupinský, J., Kaňuk, J., Hochmuth, Z., & Gallay, M. (2019). Detecting dynamics of cave floor ice with selective cloud-to-cloud approach. The Cryosphere, 13(11), 2835-2851.

Santo, A., Budetta, P., Forte, G., Marino, E., & Pignalosa, A. (2017). Karst collapse susceptibility assessment: A case study on the Amalfi Coast (Southern Italy). Geomorphology, 285, 247-259.

Idrees, M. O., & Pradhan, B. (2018). Geostructural stability assessment of cave using rock surface discontinuity extracted from terrestrial laser scanning point cloud. Journal of Rock Mechanics and Geotechnical Engineering, 10(3), 534-544.

Núñez, M. A., Buill, F., & Edo, M. (2013). 3D model of the Can Sadurní cave. Journal of archaeological Science, 40(12), 4420-4428.

Cai, W., Zhu, H., & Liang, W. (2022). Three-dimensional tunnel face extrusion and reinforcement effects of underground excavations in deep rock masses. International Journal of Rock Mechanics and Mining Sciences, 150, 104999. https://doi.org/10.1016/j.ijrmms.2021.104999

Cosso, T., Ferrando, I., & Orlando, A. (2014). SURVEYING AND MAPPING A CAVE USING 3D LASER SCANNER: THE OPEN CHALLENGE WITH FREE AND OPEN SOURCE SOFTWARE. International Archives of the Photogrammetry, Remote Sensing & Spatial Information Sciences, 45.

Van Rentergem, G., McFarlane, D., Lundberg, J., & Buchroithner, M. (2017). Lessons learned from a large-scale 3-D mapping project, with Faro laser scanners, of the Gomantong Caves, Borneo.

Idrees, M. O., & Pradhan, B. (2017). Characterization of macro-and micro-geomorphology of cave channel from high-resolution 3D laser scanning survey: case study of Gomantong cave in Sabah, Malaysia. In Cave Investigation. IntechOpen.

Idrees, M. O., & Pradhan, B. (2016b). Hybrid Taguchi-objective function optimization approach for automatic cave bird detection from terrestrial laser scanning intensity image. International Journal of Speleology, 45(3), 9.

Tan, K., & Cheng, X. (2016). Correction of incidence angle and distance effects on TLS intensity data based on reference targets. Remote Sensing, 8(3), 251.

Franceschi, M., Teza, G., Preto, N., Pesci, A., Galgaro, A., & Girardi, S. (2009). Discrimination between marls and limestones using intensity data from terrestrial laser scanner. ISPRS Journal of Photogrammetry and remote sensing, 64(6), 522-528.

Carrea, D., Abellan, A., Humair, F., Matasci, B., Derron, M. H., & Jaboyedoff, M. (2016). Correction of terrestrial LiDAR intensity channel using Oren–Nayar reflectance model: An application to lithological differentiation. ISPRS Journal of Photogrammetry and Remote Sensing, 113, 17-29.

Jalandoni A, Winans W. R., & Willis, M. D. (2021). Intensity Values of Terrestrial Laser Scans Reveal Hidden Black Rock Art Pigment. Remote Sensing, 13(7), 1357. https://doi.org/10.3390/rs13071357

Tan, K., Cheng, X., Ding, X., & Zhang, Q. (2015). Intensity data correction for the distance effect in terrestrial laser scanners. IEEE Journal of Selected Topics in Applied Earth Observations and Remote Sensing, 9(1), 304-312.

Živec, T., Anžur, A., & Verbovšek, T. (2019). Determination of rock type and moisture content in flysch using TLS intensity in the Elerji quarry (south-west Slovenia). Bulletin of Engineering Geology and the Environment, 78(3), 1631-1643.

Kashani, A. G., Olsen, M. J., Parrish, C. E., & Wilson, N. (2015). A review of LiDAR radiometric processing: From ad hoc intensity correction to rigorous radiometric calibration. Sensors, 15(11), 28099-28128.

Höfle, B., & Pfeifer, N. (2007). Correction of laser scanning intensity data: Data and model-driven approaches. ISPRS Journal of Photogrammetry and Remote Sensing, 62(6), 415-433.

Wagner, W. (2010). Radiometric calibration of small-footprint full-waveform airborne laser scanner measurements: Basic physical concepts. ISPRS Journal of Photogrammetry and Remote Sensing, 65(6), 505-513.

Teo, T. A., & Yu, H. L. (2015). Empirical radiometric normalization of road points from terrestrial mobile LiDAR system. Remote Sensing, 7(5), 6336-6357.

Xu, T., Xu, L., Yang, B., Li, X., & Yao, J. (2017). Terrestrial laser scanning intensity correction by piecewise fitting and overlap-driven adjustment. Remote Sensing, 9(11), 1090.

Soudarissanane, S., Lindenbergh, R., Menenti, M., & Teunissen, P. (2011). Scanning geometry: Influencing factor on the quality of terrestrial laser scanning points. ISPRS Journal of Photogrammetry and Remote Sensing, 66(4), 389-399.

Kaasalainen, S., Jaakkola, A., Kaasalainen, M., Krooks, A., & Kukko, A. (2011). Analysis of incidence angle and distance effects on terrestrial laser scanner intensity: Search for correction methods. Remote Sensing, 3(10), 2207-2221.

Tan, K., Cheng, X., & Cheng, X. (2016). Modeling hemispherical reflectance for natural surfaces based on terrestrial laser scanning backscattered intensity data. Optics express, 24(20), 22971-22988.

Wagner, W., Ullrich, A., Ducic, V., Melzer, T., & Studnicka, N. (2006). Gaussian decomposition and calibration of a novel small-footprint full-waveform digitising airborne laser scanner. ISPRS Journal of Photogrammetry and Remote Sensing, 60(2), 100-112.

Sorriaux, P., Guinot, R., Ferré, E., Gallay, M., Šupinský, J., de Saint Blanquat, M., & Asti, R. (2019). Expédition spéléo-scientifique au gouffre Georges: Relevé 3D de la Grande galerie. Spelunca, 5, 5-13.

Pfennigbauer, M., & Ullrich, A. (2010). Improving quality of laser scanning data acquisition through calibrated amplitude and pulse deviation measurement. In Laser Radar Technology and Applications XV (Vol. 7684, p. 76841F). International Society for Optics and Photonics.

RIEGL Laser Measurement Systems GmbH, (2017). LAS Extrabytes Implementation in RIEGL Software Whitepaper.

RIEGL Laser Measurement Systems GmbH, (2015). 3D Terrestrial laser scanner RIEGL VZ-400/RIEGL VZ-1000/RIEGL VZ-2000 General Description and Data Interfaces.

James, J. M. (2013). Atmospheric processes in caves. Shroder, J. (Editorin Chief), Frumkin, A.(Ed.), Treatise on Geomorphology. Academic Press, San Diego, CA, 6, 304-318.

Oren, M., & Nayar, S. K. (1994, May). Seeing beyond Lambert's law. In European Conference on Computer Vision (pp. 269-280). Springer, Berlin, Heidelberg.

Oren, M., & Nayar, S. K. (1995). Generalization of the Lambertian model and implications for machine vision. International Journal of Computer Vision, 14(3), 227-251.

Ding, Q., Chen, W., King, B., Liu, Y., & Liu, G. (2013). Combination of overlap-driven adjustment and Phong model for LiDAR intensity correction. ISPRS journal of photogrammetry and remote sensing, 75, 40-47.

Jelalian, A. V. (1992). Laser radar systems. Artech House.

Kim, I. I., McArthur, B., & Korevaar, E. J. (2001, February). Comparison of laser beam propagation at 785 nm and 1550 nm in fog and haze for optical wireless communications. In Optical Wireless Communications III (Vol. 4214, pp. 26-37). International Society for Optics and Photonics.

Matasci, B., Carrea, D., Abellan, A., Derron, M. H., Humair, F., Jaboyedoff, M., & Metzger, R. (2015). Geological mapping and fold modeling using Terrestrial Laser Scanning point clouds: application to the Dents-du-Midi limestone massif (Switzerland). European Journal of Remote Sensing, 48(1), 569-591.

De Waele, J., Fabbri, S., Santagata, T., Chiarini, V., Columbu, A., & Pisani, L. (2018). Geomorphological and speleogenetical observations using terrestrial laser scanning and 3D photogrammetry in a gypsum cave (Emilia Romagna, N. Italy). Geomorphology, 319, 47-61.

Triantafyllou, A., Watlet, A., Le Mouélic, S., Camelbeeck, T., Civet, F., Kaufmann, O., ... & Vandycke, S. (2019). 3-D digital outcrop model for analysis of brittle deformation and lithological mapping (Lorette cave, Belgium). Journal of Structural Geology, 120, 55-66.

Schütz, M., & Wimmer, M. (2015). Rendering large point clouds in web browsers. Proceedings of CESCG, 83-90.

Isenburg, M., 2019. LAStools - efficient tools for LiDAR processing. version 200509, http://lastools.org.

Girardeau-Montaut, D.: CloudCompare – 3D point cloud and mesh processing software. Open Source Project, available at: https://www.danielgm.net/cc/ (last access: 4 November 2020), 2018.

Lague, D., Brodu, N., and Leroux, J.: Accurate 3D comparison of complex topography with terrestrial laser scanner: Application to the Rangitikei canyon (N-Z), ISPRS J. Photogramm., 82, 10-26, https://doi.org/10.1016/j.isprsjprs.2013.04.009, 2013.

Monchoux, P. (1970), Les lherzolites Pyrénéennes: Contribution à l’étude de leur minéralogie, de leur genèse et de leurs transformations, Ph.D. thesis, 180 pp., Univ. of Toulouse, Toulouse, France.

LAGABRIELLE Y., LABAUME P. & DE SAINT BLANQUAT M. (2010). – Mantle exhumation, crustal denudation, and gravity tectonics during Cretaceous rifting in the Pyrenean realm (SW Europe): Insights from the geological setting of the lherzolite bodies. – Tectonics, 29, TC4012, doi: 10.1029/2009TC002588.

Vergés, J., M. Fernàndez, and A. Martínez (2002), The Pyrenean orogen: Pre-, syn-, and post-collisional evolution, in Reconstruction of the Evolution of the Alpine-Himalayan Orogen, vol. 8, edited by G. Rosenbaum and G. Lister, pp. 55–74, Journal of the Virtual Explorer.

Saspiturry, N., Allanic, C., Razin, P., Issautier, B., Baudin, T., Lasseur, E., Serrano, O., & Leleu, S. (2020a). Closure of a hyperextended system in an orogenic lithospheric pop-up, Western Pyrenees: The role of mantle buttressing and rift structural inheritance. Terra Nova, 32(4), 253–260. https://doi.org/10.1111/ter.12457

Lagabrielle, Y., Bodinier, J.-L., 2008. Submarine reworking of exhumed sub-continental mantle rocks: field evidence from the Lherz peridotites, French Pyrenees: cretaceous exhumation of pyrenean mantle. Terra. Nova 20 (1), 11–21. https://doi.org/10.1111/j.1365-3121.2007.00781.x.

Jammes, S., Manatschal, G., Lavier, L., Masini, E., 2009. Tectono-sedimentary evolution related to extreme crustal thinning ahead of a propagating ocean: example of the western Pyrenees. Tectonics 28, 4. https://doi.org/10.1029/2008TC002406.

Clerc, C., & Lagabrielle, Y. (2014). Thermal control on the modes of crustal thinning leading to mantle exhumation: Insights from the Cretaceous Pyrenean hot paleomargins. Tectonics, 33, 1340–1359. https://doi.org/10.1002/2013TC003471

Masini, E., Manatschal, G., Tugend, J., Mohn, G., Flament, J.-M., 2014. The tectonosedimentary evolution of a hyper-extended rift basin: the example of the Arzacq–Mauléon rift system (Western Pyrenees, SW France). Int. J. Earth Sci. 103 (6), 1569–1596. https://doi.org/10.1007/s00531-014-1023-8.

Asti, R., Lagabrielle, Y., Fourcade, S., Corre, B., & Monié, P. (2019). How do continents deform during mantle exhumation? Insights from the northern Iberia inverted paleopassive margin, western Pyrenees (France). Tectonics, 38, 1666–1693. https://doi.org/10.1029/2018TC005428

Lagabrielle, Y., Asti, R., Fourcade, S., Corre, B., Poujol, M., Uzel, J., Labaume, P., Clerc, C., Lafay, R., Picazo, S., Maury, R., (2019a). Mantle exhumation at magma-poor passive continental margins. Part I. 3D architecture and metasomatic evolution of a fossil exhumed mantle domain (Urdach lherzolite, North-Western Pyrenees, France). BSGF - Earth Sci. Bull. 190, 8. https://doi.org/10.1051/bsgf/2019007.

Lagabrielle, Y., Asti, R., Fourcade, S., Corre, B., Uzel, J., Labaume, P., Clerc, C., Lafay, R., Picazo, S., (2019b). The mechanisms of mantle exhumation at magma-poor passive continental margins. Part II. Insights from high-displacement, low-angle faults preserved in a fossil distal margin domain (Saraillé lherzolites, north-western Pyrenees, France). BSGF Earth Sci. Bull. 190, 14. https://doi.org/10.1051/bsgf/2019013.

Saspiturry, N., Razin, P., Baudin, T., Serrano, O., Issautier, B., Lasseur, E., Allanic, C., Thinon, I., & Leleu, S. (2019). Symmetry vs. asymmetry of a hyper-thinned rift: Example of the Mauléon Basin (Western Pyrenees, France). Marine and Petroleum Geology, 104, 86–105. https://doi.org/10.1016/j.marpetgeo.2019.03.03

Clerc, C., Lahfid, A., Monié, P., Lagabrielle, Y., Chopin, C., Poujol, M., et al. (2015). High‐temperature metamorphism during extreme thinning of the continental crust: A reappraisal of the north Pyrenean passive paleomargin. Solid Earth, 6(2), 643–668. https://doi.org/10.5194/se‐6‐643‐2015

Lagabrielle, Y., Clerc, C., Vauchez, A., Lahfid, A., Labaume, P., Azambre, B., et al. (2016). Very high geothermal gradient during mantle exhumation recorded in mylonitic marbles and carbonate breccias from a Mesozoic Pyrenean palaeomargin (Lherz area, North Pyrenean Zone, France). Comptes Rendus Geosciences, 348, 257–267.

Ducoux, M., Jolivet, L., Cagnard, F., Gumiaux, C., Baudin, T., Masini, E., Callot, J.P., Aubourg, C., Lahfid, A., Homonnay, E., 2019. The Nappe des Marbres unit of the Basque-Cantabrian Basin: the Tectono-Thermal Evolution of a Fossil Hyperextended rift Basin. Tectonics in presshttps://doi.org/10.1029/2018TC005348.

Duretz, T., Asti, R., Lagabrielle, Y., Brun, J.P., Jourdon, A., Clerc, C., Corre, B., 2019. Numerical modelling of syn-rift salt tectonics during Cretaceous Pyrenean Rifting. Basin Res 1–16. https://doi.org/10.1111/bre.12389.

Saspiturry, N., Lahfid, A., Baudin, T., Guillou-Frottier, G., Razin, P., Issautier, B., Le Bayon, B., Serrano, O., Lagabrielle, Y., & Corre, B. (2020b). Paleogeothermal gradients across an inverted hyperextended rift system: Example of the mauléon fossil rift (Western Pyrenees). Tectonics, 39(10), 1–36. https://doi.org/10.1029/2020TC006206.

Tavani S, Bertok C, Granado P, Piana F, Salas R, Vigna B, Muñoz JA. 2018. The Iberia-Eurasia plate boundary east of the Pyrenees. Earth-Science Reviews 187:314–337, https://doi.org/10.1016/j.earscirev.2018.10.008.

Angrand P, Mouthereau F, Masini E, Asti R. 2020. A reconstruction of Iberia accounting for W-Tethys/N-Atlantic kinematics since the late Permian-Triassic. Solid Earth, https://doi.org/10.5194/se-2020-24

Debroas, E. J., Canérot, J., & Bilotte, M. (2010). Les Brèches d'Urdach, témoins de l'exhumation du manteau pyrénéen dans un escarpement de faille Vraconnien‐Cénomanien inférieur (zone nord‐pyrénéenne, Pyrénées‐Atlantiques, France). Géologie de la France, 2, 53–63.

Clerc C, Lagabrielle, Y., Neumaier M., Reynaud J.-Y., Saint Blanquat M. de (2012) - Exhumation of subcontinental mantle rocks : evidence from ultramafic-bearing clatic deposits nearby the Lherz peridotite body, French Pyrenees - Bull. Soc. Geol. Fr.

M. De Saint Blanquat, F. Bajolet, A. Grand'Homme, A. Proietti, M. Zanti, A. Boutin, C. Clerc, Y. Lagabrielle, P. Labaume (2016).Cretaceous mantle exhumation in the central Pyrenees: new constraints from the peridotites in eastern Ariège (North Pyrenean zone, France) Compt. Rendus Geosci., 348, pp. 268-278

Cignoni, P., Callieri, M. , Corsini, M., Dellepiane, M., Ganovelli, F., Ranzuglia, G.
2008. MeshLab: an Open-Source Mesh Processing Tool. Sixth Eurographics Italian Chapter Conference, pp. 129-136.

Ullrich, A.; Schwarz, R.; Kager, H. Using hybrid multi-station adjustment for an integrated camera laser-scanner system. Opt. 3-D Meas. Tech. IV 2003, 1, 298–305.

- Rapidlasso GmbH. LAStools. Available online: https://rapidlasso.com/lastools/ (accessed on 1 August 2019).

- Potree. Available online: http://www.potree.org/. (accessed on 1 August 2019).

- Schuetz, M. Potree: Rendering Large Point Clouds in Web Browsers. Engineer Diploma Thesis, Vienna University of Technology, Vienna, Austria, 2016; p. 92. [Google Scholar]

- Scheiblauer, C. Interactions with Gigantic Point Clouds. Ph.D. Thesis, Vienna University of Technology, Vienna, Austria, 2014; p. 203. [Google Scholar]